Theoretical Energy Required  Per SWU

•  From thermodynamics

\f(kWh,SWU) = \f(4RT,M(1-α)2)

R = Gas Constant= 8.31441 \f(J,mole °K)

Where,  T = ° K

M = Molecular Mass

if T= 58° C,

min Energy/SWU = 0.0802 \f(kWy,SWU)

= 702  \f(kWh,SWU)

actually,                               ≈ 3000 \f(kWh,SWU)

-------------------------------------------------

History

US Capacity in mid 1970’s     17,000,000 \f(  SWU,y)

Cascade Improvement &       10,000,000  \f(SWU,y)    Cascade Uprating Prog

(1983) 					    ___________

total		     27,000,000 \f(SWU,y)

Because of plant cancellations in 1970’s and 1980’s, and because of European built plants, there is considerable excess capacity

Figure 2.21

Centrifuge Method

Date			Events

1938	JW Beam, Germany in WWII

1950’s	Germans built larger gas centrifuges

1960’s	Centrifuges become competitive

1970	FRG and Netherlands build a plant

1976	Japanese build a plant

1977	Carter Administration announces 	that the next  enrichment plant 		would use the gas centrifuge process.

1985	Almost completed gas centrifuge  	plant (8,800,000 SWU/y) shut down 	by Reagan administration for laser 	isotope plant.

----------------------------------

Principles

•   A rotating drum compresses gas molecules in the cylinder to outer wall

•  Lighter molecules concentrate in the center

Figure 3.7

Gas Centrifuge factor;

where;

ω = angular velocity (rad/s)

a = radius of rotor ( inside dimension)

= Molecular Wt. of

= Molecular Wt. of

for 30 cm diameter rotor at 350 \f(m,s) @ 300 °K

α = 1.1 to 1.2  ( Compare to gas diffusion)

--------------------------------

Features

•	Small capacities; a 9,000,000 \f(SWU,y) plant requires 90,000 to 100,000 machines

•	Reliability - if mean time between failure is 3 years, then more than 3 machines break per h

•	Lower Power requirements ≈ 300 \f(kWh,SWU)

if 1 kWh ≈ 5¢, then;

Gas centrifuge cost ≈ \f($15, SWU)

Gaseous Diffusion ≈ \f($ 150, SWU)

Nozzle Method

•   Developed by Becker in FRG, 1960

( later in South Africa, Netherlands)

•	UF6 (5%) + H2 (95%) @ 20 -200 torr forced 				into orifice  (See figure 3.8)

α = 1.015

•	Must be very precise and clean (≈ 30μ)

•	Smaller number of stages than gaseous 				diffusion to get to 3% (500 vs 1200)

•	Slightly higher electricity consumption

4000 \f(kWh,SWU)

-------------------------------

Atomic Vapor Laser Isotope Separation (AVLIS)

•	First demonstrated in 1975

• 	Principle relies on many absorption lines in 							U (300,000)

•	There is a significant separation between 	 and  which can be exploited by the 		use of variable wavelength lasers.

Figure 3.9

Figure 3.10

1.)	Pool of liquid U formed

2.)	Photo excite  at 5915Å

3.) 	Use UV light to ionize excited atom

4.)	Collect positively charged

Advantage - 300 \f(kWh,SWU)

plus less moving parts and higher reliability

---------------------------------------------

Overall Costs For Enriched U

Price of 		Price of	Conversion	SWU

Enriched U	Natural U	Costs			Costs

The price of enriched U can be optimized with respect to waste tails

\f(δ PE,δxw ) = 0

gives;

Problem -2

Calculate the cost of enriched U (in \f($,kg) )if the product is 2.8% enriched, the tails assay is 0.25%, and the conversion loss is 0.5%.  Assume the price of natural U is $40/kg, the cost of conversion is $6/kg, and the price is $80/SWU.

Answer:

\f(F,P) = \f((2.8-0.25),(0.7111-0.25))  = 5.531

V(xf) = (2•0.00711-1) • ln[\f(0.00711,(1-0.00711))]

= 4.869

V(xp)= (2•0.028-1) • ln[\f(0.028,(1-0.028))] = 3.348

V(xw)=(2•0.0025-1) •ln[\f(0.0025,(1-0.0025))]=5.959

S=3.348+4.531•5.959-5.531•4.869=3.418   \f(SWU,kg)

PE=[\f(40,(1-0.005)) +6] • 5.531+80 •3.418= $ 529/kg

---------------------------------------------------

Total Fabricated Costs

= fraction of U lost in conversion

= fraction of U lost in fabrication

PF = Price of Fabricated Fuel

Problem -3

What is the cost of fabricated fuel if the price for  fabrication is $200/kg and there is an 0.8% loss during fabrication.    Use the numbers from problem 2 for nat. U, conversion, and enrichments costs.

FF=  732 $/kg