Problems Due Friday, Sept. 29, 1995

1.)  An enrichment plant has a throughput of 32,000 kgU/day and produces 26,000 kgU as tails.  What is the enrichment of the product if the feed is natural U and the tails are 0.25%?

2.)  A gaseous diffusion method has been proposed to produce BF3 enriched to 90% in B10.  How many kgs of BF3 feed (natural B) are needed to produce 1 kg of enriched product with 8% tails?

3.)  Calculate the natural U feed and SWU factors 1 billion years into the future.  Assume tails of 0.15% and 3% enriched product;

t1/2 (35U) = 7.1 x 108 y,

t1/2 (38U) = 4.51 x 109 y.

4.)  Assuming that the price per SWU is $80 and the cost of conversion is $4/kgU, what is the price of the U3O8 ($lb U3O8) beyond which it will cost less to enrich the already mined , purified, and converted (to UF6) tails that contain 0.2% 35U rather than mine new U?

(Assume the product will be 3% enriched in either 	case and the new tails will be 0.1%(when the old 	tails are enriched).  Tails stored as UF6 cost 	nothing.)